Co3 AT2

Mahesh U(192312580)

33. Multi-Channel Decimation for Massive MIMO

Simulate multi-channel decimation in a Massive MIMO receiver and analyze computational

reduction. (25) (BL4) (WK4)

Given Data:

● Number of Channels = 16

● Sampling Rate = 122.88 MHz

● Decimation Factor = 4

Tasks:

1. Generate multi-channel signals.

2. Design a common decimation filter.

3. Perform decimation.

4. Analyze computational reduction.

5. Compare channel spectra.

6. Discuss implementation benefits.

Answer:

#Code

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import firwin, lfilter

# ============================================================

# MULTI-CHANNEL DECIMATION FOR MASSIVE MIMO

# ============================================================

# Given parameters

num_channels = 16

Fs = 122.88e6                 # Sampling frequency = 122.88 MHz

D = 4                         # Decimation factor

Fs_dec = Fs / D               # Decimated sampling frequency

# Simulation duration

duration = 50e-6              # 50 microseconds

N = int(Fs * duration)

# Time vector

t = np.arange(N) / Fs

# ============================================================

# 1. GENERATE MULTI-CHANNEL SIGNALS

# ============================================================

np.random.seed(1)

# Different frequency for each MIMO channel

channel_freqs = np.linspace(2e6, 14e6, num_channels)

signals = []

for i in range(num_channels):

# Complex sinusoidal signal

signal = np.exp(1j * 2 * np.pi * channel_freqs[i] * t)

# Add noise

noise = 0.1 * (

np.random.randn(N) +

1j * np.random.randn(N)

)

signal = signal + noise

signals.append(signal)

signals = np.array(signals)

print("==============================================")

print(" MULTI-CHANNEL DECIMATION FOR MASSIVE MIMO")

print("==============================================")

print("Number of Channels       :", num_channels)

print("Input Sampling Rate      :", Fs / 1e6, "MHz")

print("Decimation Factor        :", D)

print("Output Sampling Rate     :", Fs_dec / 1e6, "MHz")

# ============================================================

# 2. DESIGN COMMON DECIMATION FILTER

# ============================================================

num_taps = 63

# Anti-aliasing low-pass FIR filter

cutoff = 0.45 / D

h = firwin(num_taps, cutoff)

print("FIR Filter Taps          :", num_taps)

# ============================================================

# 3. FILTER AND DECIMATE ALL CHANNELS

# ============================================================

decimated_signals = []

for i in range(num_channels):

# Apply common FIR filter

filtered_signal = lfilter(h, 1.0, signals[i])

# Decimate by factor 4

decimated_signal = filtered_signal[::D]

decimated_signals.append(decimated_signal)

decimated_signals = np.array(decimated_signals)

print("Input Samples/Channel    :", N)

print("Output Samples/Channel   :", len(decimated_signals[0]))

# ============================================================

# 4. COMPUTATIONAL REDUCTION

# ============================================================

input_samples = num_channels * N

output_samples = num_channels * len(decimated_signals[0])

sample_reduction = (

1 - output_samples / input_samples

) * 100

print()

print("Total Input Samples      :", input_samples)

print("Total Output Samples     :", output_samples)

print("Sample Reduction         :",

round(sample_reduction, 2), "%")

# Approximate filtering operations

operations_before = num_channels * N * num_taps

operations_after = num_channels * (N / D) * num_taps

computational_reduction = (

1 - operations_after / operations_before

) * 100

print("Estimated Computational")

print("Reduction                :",

round(computational_reduction, 2), "%")

# ============================================================

# 5. FUNCTION TO CALCULATE SPECTRUM

# ============================================================

def calculate_spectrum(signal, Fs):

N = len(signal)

window = np.hanning(N)

spectrum = np.fft.fft(signal * window)

spectrum = np.fft.fftshift(spectrum)

frequency = np.fft.fftfreq(N, 1 / Fs)

frequency = np.fft.fftshift(frequency)

magnitude = 20 * np.log10(

np.abs(spectrum) /

np.max(np.abs(spectrum)) + 1e-12

)

return frequency, magnitude

# ============================================================

# 6. COMPARE CHANNEL 1 SPECTRUM

# ============================================================

f_before, spectrum_before = calculate_spectrum(

signals[0], Fs

)

f_after, spectrum_after = calculate_spectrum(

decimated_signals[0], Fs_dec

)

plt.figure(figsize=(10, 5))

plt.plot(

f_before / 1e6,

spectrum_before,

label="Before Decimation"

)

plt.plot(

f_after / 1e6,

spectrum_after,

label="After Decimation"

)

plt.xlim(0, 60)

plt.ylim(-80, 5)

plt.xlabel("Frequency (MHz)")

plt.ylabel("Magnitude (dB)")

plt.title(

"Channel 1 Spectrum Before and After Decimation"

)

plt.grid(True)

plt.legend()

plt.show()

# ============================================================

# 7. PLOT SPECTRA OF ALL 16 CHANNELS

# ============================================================

plt.figure(figsize=(10, 6))

for i in range(num_channels):

f, spectrum = calculate_spectrum(

decimated_signals[i],

Fs_dec

)

plt.plot(

f / 1e6,

spectrum,

label="Channel " + str(i + 1)

)

plt.xlim(0, Fs_dec / 2 / 1e6)

plt.ylim(-80, 5)

plt.xlabel("Frequency (MHz)")

plt.ylabel("Magnitude (dB)")

plt.title(

"Spectra of 16 MIMO Channels After Decimation"

)

plt.grid(True)

plt.legend(

ncol=4,

fontsize=8

)

plt.show()

# ============================================================

# 8. DISPLAY IMPLEMENTATION BENEFITS

# ============================================================

print()

print("==============================================")

print(" IMPLEMENTATION BENEFITS")

print("==============================================")

print("1. Common FIR filter can be used for all channels.")

print("2. Sampling rate reduced from 122.88 MHz to 30.72 MHz.")

print("3. Data volume reduced by 75%.")

print("4. Memory bandwidth requirement is reduced.")

print("5. Downstream DSP processing becomes faster.")

print("6. FPGA/ASIC implementation becomes more efficient.")

print("7. Polyphase FIR can further improve hardware efficiency.")

Output: